江 苏 坤 泰 机 械 有 限 公 司

JIANGSU KUNTAI MACHINERY CO.,LTD

KT-FT-2000

Solvent Based Glue Lamination Machine

Configuration

Jiangsu Kuntai Machinery Co., Ltd

- Major Technical Parameters

Machine roller width: 2000mm

Max material width: 1800mm

Machine speed: 20-40m/min

Installed Power: About 58kw

- Configuration

- Hydraulic Edge Alignment Fabric Feeding Device

- Hydraulic edge alignment system                  1 set

- 2000 Tension apparatus                          2 set

- Φ60 Stainless steel covered fabric guiding roller       3pcs

- Infrared U-shaped detector                       1pcs

- b. Gluing device

- Footboard                                      1pcs

- Φ240 Glue transfer roller                          1pcs

- Φ200 Silicone roller                               1pcs

- Φ150 Silicone roller                               1pcs

- Φ125 Aluminum fabric distribution roller              1pcs

- 2050 Square tube cutter body                       1pcs

- 2050 Blue steel blade                              3pcs

- Stainless glue washing channel                      1pcs

- Φ800×80k Cylinder                               2pcs

- Φ80×60k Cylinder                               2pcs

- 3kw Less swing motor                            1pcs

- 3kw Inverter                                   1pcs

- Φ60 Stainless steel covered fabric guiding roller       3pcs

- c. Lamination Device

- Φ1500 Drying roller                               1pcs

- Φ150 Silicone roller                               4pcs

- Φ80×50 Cylinder                                 8pcs

- 2000 Tension apparatus                            1pcs

- 22219 Bearing                                    2set

- 3kw Less swing motor                             1pcs

- 3kw Inverter                                     1pcs

- Pneumatic Edge Alignment                         2set

- Infrared U-shaped detector                         2pcs

- Magnetic Valve                                  2set

- Conveyor Belt                                   1set

- 1.5kw Motor                                    1pcs

- 1.5kw Inverter                                   1pcs

- Centering Device                                 1set

- Edge stretcher                                   2set

- Air pump                                       2set

- Distributor                                      2set

- d. Cooling Device

- Φ504 Cooling roller                                2pcs

- Φ150 Rubber roller                                1pcs

- Φ80×50k Cylinder                                 2pcs

- P2 Tooth chain CVT                                1pcs

- Rotary joint                                      4pcs

- Metal Hose                                      4pcs

- Φ60 Stainless steel covered fabric guiding roller         3pcs

- e. Linkage synchronous cutter device

- Φ140 Blade                                      4pcs

- Synchronous cutter device                          1set

- Auto edge waste remover                           2set

- 3kw air pump                                     2pcs

- f. Guiding Device

- Φ120 Guiding roller                               1pcs

- Φ120 Rubber roller                               1pcs

- Φ63×50k Cylinder                                2pcs

- Counting device                                  1set

- Φ60 Stainless steel covered fabric guiding roller        5pcs

- g. Rewinding Device

- Φ215 Rewinding steel roller                        2pcs

- Edge guide device                                1pcs

- 1.5kw Less swing motor                           1pcs

- 1.5kw Inverter                                   1pcs

- Fabric Roll Lifting device (Motorised) with hook         1set

- h. TPU Film Unwinder and waste film rewinder

- i. Machine rack: Welded by square tubes and so on.

- j. Electrical control and operation panel

- Main Control Box: 1.5mm welded upright box

- Main control box controls motors, guiding and so on.

- Electrical Appliance Brand: Schneider

- Regional Operation Panels: Placed on some parts of the machine and on the panel, there are signal light, buttons, buzzer, motor tachometer and so on.

Note：

Under certain circumstances, there might be slight difference (The accessories brand will not be changed) in the structure and design, which will not affect the performance of the configuration.